G-Code Ideation

| Command | Description | Example | Example Description |
| --- | --- | --- | --- |
| G0 | Rapid Movement | G0 R100 T100 Z100 | Positioning 100 mm |
| G1 | Linear Movement | G1 R-10000 T-10000 Z90000 FR450000000 FT450000000 FZ25000000 | Movement at feed rate speed (Can define feed rate after axis definition) Set feed to 100 |
| G4 | Wait | G4 P10 | Wait 10 sec |
| G5 | Wait until RPM reached (CUSTOM) | G5 | Continues only when desired A-axis RPM steady state |
| G6 | Wait until print completion is recognized (metrology) | G6 |  |
| G33 | Continuous Rotational Movement | G33 A1000 | Set A axis speed to 1000 rpm |
| F | Feed Rate | F100 | Feed rate at 100 mm/sec |
| G28 | Homing | G28 | Returns the projector to home coordinates |
| M17 | Motors Enable | M17 |  |
| M18 | Motors Disable | M18 | To be used at the end of script for ALL |
| M112 | Emergency Stop | M112 | For stopping the prints abruptly. |
| M200 | Projector On |  |  |
| M201 | Projector Off |  |  |
| M202 | Projector Video Play |  |  |
| M203 | Projector Video Pause |  |  |
| M204 | Projector Video Restart |  |  |
| G90/91 | Absolute/Relative Positioning | G90/G91 |  |
| G92 | Set current position to 0 | G92 X | Set current x-axis position to 0 |

THIS IS FOR THE CURRENT CAL SYSTEM, WHERE TRANSVERSE MOTION ISN’T OCCURRING YET (NO FEED RATE REQUIRED AT THIS MOMENT)

General Start sequence :-

M17

G28

G0 X[Midpoint] Y[Desired 0] Z[Desired 0]

G92

G33 A9

G5

In current Gcode generation

G4 P10000

With the metrology system feeding back to the projector,

G6

General End sequence -

G33 A0

G28

M18